- Before Start:

  - Remove Caps pin and adjust placard;

  - Put the fuel selector to the left tank;

  - Verify parking break is set;

  - Turn on Bat 1 and 2;

  - Turn on Nav and Strobe Lights;

  - Turn on avionics;

  - Verify voltage is within 23 and 25 volts;

  - Check MDF date is not expired;

  - Announce passenger briefing is complete;

  - Set the fuel level in the MDF;

  - Set the weights and balance and press confirm;

  - Set/verify 118.35 is in COM2;

  - Listen to the ATIS information and take notes;

  - Set the altimeters with Atis information;

  - Turn avionics off;

  - Check left, center, Right; Announce Clear prop;

  - Close the door;

  - Announce and Run the checklist;

- Engine Start:

  - Insert Key;

  - Hold you breaks;

  - Set power lever to ¼;

  - Prime Mixture as required;

  - Set mixture cut off;

  - Turn the keys and push mixture full rich;

  - Verify oil pressure;

  - Set RPM to 1200;

  - Lean mixture;

  - Set RPM to 1000;

  - Turn Alternator 1 and 2 on;

  - Turn avionic on;

  - Verify volts is 28 and charging battery 1;

  - Verify Engine parameters;

  - Fuel selector to right tank;

  - Turn off the fuel Pump;

  - Set flaps to 50%;

  - Check the auto pilot;

  - Check the trim;

  - Set trim for take off;

  - Box check of flight controls and rudder;

  - Announce and run the checklist;

- Taxi:

  - Release parking break and turn landing light on;

  - Start taxing and check your breaks;

  - Announce Clear left, center, right;

  - During Taxi, Check HSI turns with aircraft with adequate rate of turn indicated;

  - Verify cheese indicator turn with rudder movement;

  - Verify the magnetic compass agrees with HSI;

  - Verify zero airspeed and zero vertical speed;

  - Verify attitude is no more than 5 degrees;

- Engine runup:

  - Set parking break and turn landing light off;

  - Verify doors are latched;

  - Set MFD to Engine page;

  - Announce engine runup;

  - Hold breaks;

  - Set mixture full rich;

  - Set RPM to 2200;

  - Announce alternator check and turn landing light and pitot heat on;

  - Check and announce voltage in green;

  - Turn pitot heat and landing light off;

  - Announce magneto check;

  - Announce drop and differential;

  - Announce full power check;

  - Set power lever full and announce max rpm, should be more than 2600;

  - Announce idle check;

  - Power lever to idle and announce idle power; min 700;

  - Set RPM to 1000;

  - Announce and run checklist;

- Before takeoff:

  - Perform departure briefing;

  - Confirm or set transponder 1200;

  - Set altitude bug;

  - Set runway heading;

  - Turn the fuel pump on;

  - Confirm flaps are 50%;

  - Turn off recirc;

  - Announce and run checklist;

- After takeoff:

  - Verify fuel pump is on;

  - Verify climb power is full lever;

  - Lean mixture as required;

  - Verify and announce engine is in the green;

  - Clean flaps above 85knots and 200agl;

  - Climb at 96knots

  - Announce and run checklist;

- Climb:

  - Fuel pump stays on;

  - Set climb to Vy 105 knots;

  - Climb power is full;

  - Continue to adjust the mixture to stay top of green;

  - Verify engine parameters are green;

- Cruise:

  - Level plane off and allow it to gain speed to 120;

  - Lean cyan line;

  - Turn off fuel pump;

  - Turn off landing light;

  - Announce and run cruise checklist;

- Descent:

  - Get destination weather;

  - Set altimeters;

  - Perform arrival briefing with landing plates;

  - Landing lights and Ice lights on;

  - Check brakes pressure and ask passenger to check theirs;

  - Announce and run checklist;

- Landing:

  - Seatbelts on;

  - Fuel pump on;

  - Mixture as required;

  - Disengage auto pilot;

  - Bring flaps full;

- After landing:

  - Fuel pump off;

  - Mixture lean for taxi;

  - Flaps up;